COMMONWEALTH OF VIRGINIA VIRGINIA DEPARTMENT OF ENERGY MINED LAND REPURPOSING

3405 MOUNTAIN EMPIRE ROAD; BIG STONE GAP, VA 24219 TELEPHONE: (276) 523-8100

EXCESS SPOIL FILLS AND REFUSE EMBANKMENTS CONSTRUCTION CERTIFICATION

| COMPANY NAME |  |  |  |  |  |  |  |  |  | PERMIT NO. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fill No. |  |  | Type of Fill |  |  |  |  |  | MLR Fill Id. No. |  |  |  | MPID at toe |  |  |  |  |  |
| Type of construction certification: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Current Location of Toe of Fill (NAD83 Virginia South State Plane Coordinates) |  |  |  |  | Northing |  |  |  |  |  |  | Easting |  |  |  |  |  |  |
| Removal of Organic Materials and Topsoil |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distance in feet that clearing and grubbing operations precede spoil placement. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distance in feet that topsoil removal operations precede spoil placement. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Where applicable, enter the appropriate response to the following. If not applicable, enter “NA”. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Topsoil is removed to competent subsoil or rock. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | All topsoil is being stockpiled or placed on completed portions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Placement of Underdrain System (attach color photographs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Height by width dimensions (in feet) of the underdrain per the approved detailed plans. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Actual constructed height by width dimensions (in feet) of the underdrain. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Durable rock is free of shale, fines, and other contaminants. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The rock grading is in accordance with the approved design. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Rock is being placed by selective handling from the toe of the fill. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The keyway cut was constructed in accordance with the approved design. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The rock toe buttress was constructed in accordance with the approved design. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The filter systems for the underdrain(s) were constructed in accordance with the approved design. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Lateral drains were placed to all springs and potential seeps. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Installation of the Surface Drainage System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sediment ponds were installed prior to any fill construction disturbance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Temporary diversion ditches, if applicable, were installed in accordance with the approved design. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Permanent diversion ditches, side drains and terraces are installed in accordance with the approved designs, and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - are placed on the proper grade(s). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - are constructed in accordance with the approved design dimensions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Indicate the type of erosion protection techniques used in the side drains, diversion ditches, and terraces by entering “R” for rock rip rap; “B” for bed rock; “V” for vegetation; and “O” for other (specify ). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Placement and Compaction of Materials |  |  |  |
| --- | --- | --- | --- |
|  |  | The material is placed in 4 feet lifts or less as specified in the approved design. |  |
|  |  | The haulage vehicles are routed to achieve compaction. |  |
|  |  | List other methods of compaction utilized in the fill construction. |  |
|  |  | What is the percent of compaction? |  |
|  |  | What is the moisture content of the material at the time of placement? |  |
|  |  | Potentially toxic or acid forming material is being handled according to the approved plans. |  |
|  |  | This is a durable rock fill composed of 80% durable rock, and |  |
|  |  | - with the clay or shale dispersed properly. |  |
|  |  | - the spoil is dumped at the approved locations. |  |
|  |  | - the bench height above the completed portions are no greater than 50 feet. |  |
| Vegetation |  |  |  |
|  | Topsoiling and seeding are conducted concurrently with terrace completion. |  |  |
|  | Seeding is in accordance with the approved mixtures and rates. |  |  |
|  | Indicate the approximate percentage of ground cover. |  |  |
| Sketch of Fill(s) and Support Structure(s) |  |  |  |
| Submit a sketch (on a separate sheet) of each fill and supporting structures which are subject to this certification. Include the following information: - approximate North. - outline of the fill area. - location of sediment control structure(s). - number and location of completed terraces. - the limits of organic material removal. - the limits of topsoil removal. - the location of any surface or ground water discharges. - the location of all underdrains and diversion ditches. - discuss any changes to be made, variations from the approved design, and any improper practices encountered. |  |  |  |
| Comments |  |  |  |

| Additional Data for Final Fill Certification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Attach pdf and dwg files (must include original and final contour lines) and digital photos. |  |  |  |  |  |  |
| Total volume (mcy) |  |  |  | Footprint (acres) |  |  |
| Describe methodology used to determine final fill volume. |  |  |  |  |  |  |
| Linear feet of stream covered by fill |  |  |  |  |  |  |
| Ephemeral (ft) |  | Intermittent (ft) | Perennial (ft) |  | Total covered (ft) |  |

| Certification by Registered Professional Engineer |  |  |  |
| --- | --- | --- | --- |
| I certify that the aforementioned fill is constructed and installed in accordance with the requirements of the Virginia Coal Surface Mining Reclamation Regulations and as per the approved design(s). |  |  |  |
| CERTIFIED BY: |  | Certification/Registration No. |  |
| SIGNATURE: |  | Date: |  |
| PLACE SEAL HERE |  |  |  |